# GEOTECHNICAL ENGINEERING EXAM

Department of Civil Engineering

__________________________________________________

### **Soil Mechanics Fundamentals (Professional Competence Examination)**

**Duration:** 3 Hours

**Total Marks:** 100

**Instructions:**

1. This is an open-book examination. You may refer to specified codes and handbooks (e.g., Eurocode 7, ASTM standards).

2. Show all calculations clearly. Partial credit will be awarded for correct methodology.

3. Use SI units unless otherwise specified.

---

### **Section A: Short Answer (Theory & Concepts)**

**(5 Questions × 4 Marks = 20 Marks)**

Question 1 (4 points):

Explain the principle of effective stress and its significance in geotechnical engineering.

Question 2 (4 points):

Differentiate between compaction and consolidation in terms of process, purpose, and outcomes.

Question 3 (4 points):

Describe the role of a sheepsfoot roller in soil compaction. For which types of soil is it most effective, and why?

Question 4 (4 points):

Interpret the critical state line (CSL) in shear strength theory. What does it represent in terms of soil behavior?

Question 5 (4 points):

Identify the stress components acting on a soil element. Explain their relevance in analyzing soil stability.

---

### **Section B: Analytical Problems (Calculations)**

**(4 Questions × 10 Marks = 40 Marks)**

Question 6 (10 points):

A layered soil system consists of a 3 m thick sand layer (γ = 18 kN/m³, saturated γ = 20 kN/m³) overlying a 5 m thick clay layer (γ = 16 kN/m³, saturated γ = 19 kN/m³). The water table is located 2 m below the ground surface. Calculate:

a) The total vertical stress, pore water pressure, and effective vertical stress at depths of 2 m, 4 m, and 6 m.

b) Discuss the implications of effective stress changes on soil strength.

Question 7 (10 points):

A direct shear test on a sandy soil sample yields the following results:

Normal stress: 100 kPa, 200 kPa, 300 kPa

Shear stress at failure: 50 kPa, 100 kPa, 150 kPa

Determine the soil’s shear strength parameters (c', φ'). Plot the Mohr-Coulomb failure envelope and interpret the results.

Question 8 (10 points):

A 6 m thick clay layer is subjected to a uniform load of 50 kPa. The clay has the following properties:

Compression index (Cc): 0.25

Initial void ratio (e₀): 0.8

Coefficient of consolidation (Cv): 2 × 10⁻⁴ m²/s

Preconsolidation pressure: 75 kPa

Calculate:

a) The primary consolidation settlement.

b) The time required for 90% consolidation if the drainage occurs from both top and bottom surfaces.

Question 9 (10 points):

A rectangular footing (2 m × 3 m) is placed on a sandy soil with the following properties:

Unit weight (γ): 18 kN/m³

Cohesion (c'): 0 kPa

Angle of internal friction (φ'): 30°

Depth of footing: 1.5 m

Using Terzaghi’s bearing capacity equation, calculate the ultimate bearing capacity of the footing. Assume groundwater is at a depth of 3 m. Use the following factors:

Nc = 30.14, Nq = 18.4, Nγ = 15.1

---

### **Section C: Design & Application**

**(2 Questions × 15 Marks = 30 Marks)**

Question 10 (15 points):

Design a vertical drain system for a 10 m thick soft clay layer to achieve 90% consolidation within 1 year. The clay has the following properties:

Coefficient of consolidation (Cv): 1 × 10⁻⁴ m²/s

Coefficient of permeability (k): 1 × 10⁻⁷ m/s

Drain diameter: 0.1 m

Soil smear zone diameter: 0.3 m

Determine the required spacing and pattern of the drains. Reference ASTM D7498 for design principles.

Question 11 (15 points):

A cantilever retaining wall is designed to retain a 4 m high backfill of sandy soil with the following properties:

Unit weight (γ): 18 kN/m³

Angle of internal friction (φ'): 35°

Cohesion (c'): 0 kPa

The wall has a base width of 2.5 m and a heel length of 1.5 m.

a) Calculate the active earth pressure acting on the wall.

b) Perform a stability analysis for sliding and overturning.

c) Suggest modifications to improve stability if required.

---

### **Section D: Integrated Analysis (Case Study)**

**(1 Question × 10 Marks = 10 Marks)**

Question 12 (10 points):

A slope failure occurred in a 10 m high embankment constructed on a soft clay foundation. The failure was attributed to excessive pore water pressure and inadequate shear strength. Analyze the failure by addressing the following:

a) The role of effective stress and pore pressure in slope stability.

b) The applicability of limit equilibrium methods versus advanced constitutive models (e.g., Cam-clay) in analyzing the failure.

c) Propose remediation measures to stabilize the slope and prevent future failures.

---

### **Marking Scheme**

**Section A:** Award full marks for concise, accurate answers. Partial credit for partially correct explanations.

**Section B:** Award marks for correct methodology, intermediate steps, and final answers.

**Section C:** Emphasize clarity in design assumptions, calculations, and justifications. Award partial credit for reasonable approaches.

**Section D:** Evaluate critical thinking, integration of concepts, and professional judgment.

**End of Examination**